ICAD HONO-NO2 Analyzer SERIES 210

Patented, fast, accurate and direct HONO and No2 Detection

| ICAD-HONO/NO2-210L series featuring 19" rack housing and OLED display. | WI-FI LOW MAINTENANCE LOW POWER CONSUMPTION WI-FI LOW MAINTENANCE LOW POWER CONSUMPTION DIRECT HONO & NO2 DETECTION - Iterative Cavity DOAS (ICAD) spectroscopy - High resolved spectroscopic HONO & NO2 detection from ~350 to 390nm - No observed interferences, e.g. to water, NOx, HCHO O2 - No sample drying, e.g. Nafion tubing, required - No chemicals or gases required LONG-TERM STABILITY AND ROBUSTNESS - Insensitive to temperature and vibrations - Independent from absolute light source intensity - No zero-point drift, integrated zero measurement EASY SET-UP AND OPERATION, LOW MAINTENANCE - No gas calibration required, minimal maintenance effort - Controllable via Wi-Fi devices (PC, tablet, smartphone) - Multiple data communication protocols - 19” rack - Sustainable: Long life time, low power consumption SERIE 210 NEW FEATURES - System health GUI and on-board diagnosis - Easy adjustment of span calibration in GUI - Enhanced time resolution of 1 second - Improved long term stability |
| --- | --- |
| 2800 sec 2800 sec 7.5 ppt 7.5 ppt Time scaled modified Allan deviation for HONO precision. Note: Detection limits are defined as twice the modified Allan deviation values. |  |
| Patents: EP3329251; D502016003239.4; GB3329251; FR3329251; US10473583B2; ZL201680057099.6; DE102015000423 (pending); EP325928; D502016014735.3; GB3325928; FR3325928 |  |

| ICAD Product webpage | Contact Airyx |
| --- | --- |

| GRAPHICAL USER INTERFACE AND ON-BOARD DIAGNOSIS |  | MEASUREMENT PRINCIPLE AND GAS FLOW |
| --- | --- | --- |
| The Graphical User Interface provides easy operation and quick access to advanced functions and data time series. The ICAD can be accessed and controlled by any Wi-Fi device (PC, tablet, smartphone) without additional software/apps required. A comprehensive system health table shows the health status of critical sensor data (e.g. gas flow, supply voltage, temperature) for easy and fast on-board diagnosis. |  | ICAD instruments are based on optical resonators which provide here long absorption paths of up to 3 km for precise HONO and NO2 detection especially at low concentrations. Gas concentrations are derived by patented Iterative Cavity DOAS (ICAD) spectroscopy between ~350 to 390nm. Below, the measurement scheme is shown. |

INSTRUMENT Specifications

|  |  | HONO |  | NO2 |  | Other detectable gases | CO2 (optional NDIR sensor) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Measurement range1 |  | 0 – 500 ppb |  | 0 – 2000 ppb |  | Mechanical stability | Insensitive to vibrations |  |
| Limit of det.1 (2σ) at 60 sec |  | 0.08 ppb |  | 0.16 ppb |  | Power consumption | Less than 40 W at 12 V (typ.) |  |
| Precision (1σ) | at 1 sec | 0.3 ppb |  | 0.6 ppb |  | Start-up time | Less than 1 min (typ.) |  |
|  | at 60 sec | 0.04 ppb |  | 0.08 ppb |  |  |  |  |
| Zero Drift2 ppb / month |  | < 0.2 |  | < 0.3 |  | Temp. range of operation | -10 to +40°C |  |
| Detection of NO2 / HONO |  | Direct spectroscopic measurement |  |  |  | Temperature sensitivity | Less than 0.01 ppb/°C |  |
| Response time (10% to 90%) |  | 1 sec at 2 l/min (flow adjustable) |  |  |  | Cross sensitivity | No significant cross sensitivity5 |  |
| Time resolution |  | 1 sec minimum temporal averaging |  |  |  | Weight | Less than 12 kg (depending on config) |  |
| Sample flow |  | 1 to 2 l/min |  |  |  | Consumable gases | No gases/ chemicals needed for operation |  |
| Path length characterization |  | Helium (every 1 to 2 years, optional) or NO2 reference measurement or Gas free ICOM3 method |  |  |  | Dimensions W x H x D | 19” rack housing | 43.8 x 13.3 (3HU) x 66.5 cm |
| Calibration |  | HONO or NO2 calibration gas not needed4; |  |  |  | Data communication | LAN/Wi-Fi/RS232/M2M/OPCUA; Bayern-Hessen Protocol; Volt./Current Output |  |
|  |  |  |  |  |  | Processing unit | Internal embedded PC, with data analysis and measurement software |  |

[1] Custom configurations / measurement ranges available. By reducing the measurement range better precision and LOD can be achieved.

[2] Upper limit. Regularly automated zero measurement reduce measurement noise and guarantee also a negligible zero drift.

[3] ICOM (upcoming with software update): Integrated Calibration by means of optical Modulation. Patented gas free calibration method allows consumable free, automatic path length monitoring and calibration of ICAD measurement cells. Patents: EP325928; D502016014735.3; GB3325928; FR3325928

[4] Literature absorption data for HONO and NO2 is used for gas quantification.

[5] No significant spectroscopic cross sensitivity to: Carbon oxides, Formaldehyde, Ozone, Methane, Hydrogen, Sulphide, Sulphur dioxide, Chlorine, Chlorine dioxide, Hydrogen cyanide, Hydrogen chloride, Phosphine, Hydrogen, Ammonia, Acetylene, Nitromethane, Ethylene, Ethanol, Methyl mercaptan, Ethyl mercaptan.